SOFTWARE REQUIREMENTS SPECIFICATION

FOR

Radio Communications

Document No:

Version: 1.0

Date of Submission: 19/03/2024

Prepared by

SaNa Software Service Pvt Ltd

Hyderabad

| Prepared by | Verified by | Approved by |
| --- | --- | --- |

Abstract This document describes Software Requirements for Radio Communication of TCAS.

CHANGE HISTORY

| S.No | Name of the Document | Date | Reason for changes | Version |
| --- | --- | --- | --- | --- |
| 1. | Software requirements specification for Radio communications | 19/03/2024 | Initial Version | 1.0 |

Glossary of terms

| S.No | Abbreviation | Meaning |
| --- | --- | --- |
|  | CBC | Cypher block chaining |
|  | CE | European conformity |
|  | CENELEC | European Committee for Electro Technical Standardization |
|  | CTS | Clear to send |
|  | dbm | Decibel- milli watts |
|  | D&D | Design and development |
|  | E1 | E1 is a digital transmission link with a total transmit and receive rate of 2.048 Mbps |
|  | EMI | Electromagnetic Interference |
|  | EMI/EMC | Electromagnetic Interference/Electromagnetic compatibility |
|  | EMU | Electric Multiple Unit |
|  | FMECA | Failure mode effects and criticality analysis |
|  | FSB | Full service brake |
|  | FSS | First Stop Signal |
|  | FTA | Fault tree analysis |
|  | GNSS | Global Navigation Satellite System |
|  | GPS | Global Positioning System |
|  | GSM | Global System for Mobile Communication |
|  | I&A | Identification and Authentication |
|  | I&C | Installation and commissioning |
|  | IBS | Intermediate Block Signal |
|  | IDL | Integrated Data Logger |
|  | IEC | International electro technical committee |
|  | IP | Ingress protection |
|  | IR | Indian Railways |
|  | ISA | Independent Safety Assessor |
|  | ISO | International Standards organization |
|  | IV&V | Independent Verification and Validation |
|  | KA | Authentication key |
|  | KMPH | Kilometers Per Hour |
|  | KMS | Key Management System |
|  | KS | Session Key |
|  | LC Gate | Level Crossing Gate |
|  | LE | Light engine |
|  | Loco | Locomotive |
|  | LS | Limited Supervision |
|  | LSS | Last Stop Signal |
|  | MAC | Message authentication code |
|  | MTBWSF | Meant time between wrong side failures |
|  | MTTR | Mean time to repair |
|  | NA or N/A | Not applicable |
|  | NB | Normal brake/Normal braking |
|  | NMS | Network management system |
|  | NTP | Network Time Protocol |
|  | OCIP | Operation Cum Indication Panel |
|  | OFC | Optical Fiber Cable |
|  | PHA | Preliminary Hazard Analysis |
|  | PPM | Parts per million |
|  | QA | Quality Assurance |
|  | RAMS | Reliability Availability Maintainability and Safety |
|  | RDSO | Research design and standards organization (of Indian railways) |
|  | RF | Radio Frequency |
|  | RL | Random number loco |
|  | RS | Random number station |
|  | RTC | Real time clock |
|  | RTS | Request to send |
|  | SE | Signal engineer |
|  | SIL | Safety Integrity Level |
|  | SINAD | Signal-to-noise and distortion ratio |
|  | SL | System level requirement |
|  | SM | Station master |
|  | SR | Safety requirement |
|  | TCAS | Train collision avoidance system |
|  | THR | Tolerable hazard rate |
|  | TIN | Track Identification Number |
|  | TSR | Temporary Speed Restriction |
|  | TSRMS | Temporary Speed Restriction Management System |
|  | UHF | Ultra High Frequency |
|  | USB | Universal Serial Bus |
|  | 2oo3 | 2 out of 3 |
|  | 2oo2 | 2 out of 2 |

Table of Contents

1.	Radio Communication	6

Overview	6

2.	RMIM Requirements	7

3. Comm Task - Common Features	11

Interaction with KMS	11

Management of Sessions	11

Session Descriptors	11

Communications Failure Timer	12

Session Timer	12

Packet Transmission	12

Packet Reception	13

Modem Management	14

4. Comm Task (STCAS) Requirements	15

Initialization / Configuration	15

Packet Transmission	15

Access_Authority packets	15

Station2Loco Regular Packets	16

Packet Reception	17

Block-section packets	17

Loco2Station Regular Packets	17

5. Comm Task (LocoTCAS) Requirements	18

Initialization / Configuration	18

Packet Transmission	18

Block Section Packets	18

Loco2Station Regular Packets	19

Packet Reception	19

Block Section Packets	19

Access_Authority packets	20

Station2Loco Regular Packets	20

KMS	22

Tx / Rx of Packets by LocoTCAS	23

7. protEm	25

# Radio Communication

# Overview

The Radio Communication stack comprises of the following components:

  - The Radio Protocol component part of the Communication Task is primarily responsible for encoding of outgoing radio packets and validation / decoding of incoming packets; all the requisite parameters encoding for the outgoing packets shall be received; conversely, the parameters extracted from incoming radio packets, after executing all the requisite validation procedures, shall be sent to Vital Computer.

  - The Communication Task also maintains “sessions”: in the LocoTCAS, a single session is maintained with the registered STCAS entity, while in STCAS a session is maintained for each of the LocoTCAS entities currently registered with the same.

The session is employed for:

- maintaining the session key used for secure communications

- tracking the integrity of the radio connectivity

  - The Radio Modem Interface Module (RMIM) component is responsible for the operations required for the physical transfer of radio packets over the radio modems.  RMIM shall NOT process the contents of incoming / outgoing packets (except to the extent of separating the incoming data stream into packets based on the packet delimiter) - all the processing of the vital packets shall be carried out only in the Vital Computer.

  - The KMS component is responsible for obtaining the Authentication Keys from the (central) KMS Server. The Key thus obtained are passed on to the Radio Protocol in the Vital Computer (wherein the same are stored as the Authentication_Key_Store) and used for the generation of CBC-MAC values based on which LocoTCAS and STCAS entities authenticate each other.

  - In STCAS, the processing related to Radio Protocol shall be as follows:

Initialization of the RMIM with the configuration parameters:

  - Management of timers for communication and session timeouts

  - In Loco TCAS, the processing related to Radio Protocol shall be as follows:

    - Initialization of the RMIM with the configured broadcast frequency (f0).

    - Upon successful registration with an STCAS entity (as indicated by SIL4), initialization of RMIM with the FDMA frequencies and the TDMA slot for Tx of regular packets.

    - Upon receipt of a Station2Loco Regular packet that passes the CRC check, and DESTN_LOCO_ID matches self-Loco, pass on the decoded parameters from the same to SIL4 - all other packets are to be discarded.

    - Generation of outgoing packets entirely on the basis of the messages received from SIL4.

    - Management of timers for communication and session timeouts.

  - Dual Modems. In both STCAS and LocoTCAS, two modems shall be functioning in a hot-standby mode.

# Hardware

## 2.1 Controller Board

## STM32H750

## 2.2	Serial Radio Modem

## Gaurdian 400

## 2.3 Interfaces

RS422, RS232, CAN

# RMIM Requirements

The RMIM performs the following functions:

  - Performs the initialization required for the serial port devices to which the modems are connected.

Manual Mode:

- Configuration:

  - Utilize Guardian programmer software for manual user configuration.

  - Use the setup port (RS232) in Radio Modem for configuration purposes.

  - The COM port is designated for data communication.

- Configuration Steps:

  - Establish the appropriate connections.

  - Click on the "Link Configuration" option to configure RF Link details, Modem Settings, COM Port Serial standard and Tx Activation Mode by default.

- Antenna Setup:

  - Use locomotive antennas (SW-LMA-400) for data transmission and reception.

  - Performs diagnostics checks on the modems to determine the health of the same.

Power LED Indication:

- Green: Indicates that the Guardian modem is ready and operating normally.

- Red: Indicates a hardware fault in the Guardian modem.

Status LED Indication:

- Green: Indicates that the Guardian modem is performing normal operations with no faults.

- Red: Indicates a fault in the Guardian modem.

By using the status and power LEDs, the health of the Guardian radio modem can be effectively monitored and diagnosed.

  - Frequency Switching. RMIM transmits and receives data as per a combination of FDMA and TDMA as summarized in the following:

        - All the data transmissions are synchronized to the 2-second frame-cycle that starts at exactly midnight.

        - The frame-cycle is organized as a set of 68 time-slots numbered P1 through P68, with each time-slot commencement time as indicated in the chart.

  - In each frame-cycle, RMIM shall switch the Tx and Rx frequencies of the modems at the defined points in time (identified in the chart).

  - Packet Transmission

  - A radio packet shall always be transmitted in / from a specific time-slot indicated by the Comm Task (upper-layer in Safety Computer); in the case of LocoTCAS, the packets sizes are limited to the width of a time-slot and as such the transmission of each is completed within the time-slot; however, in STCAS, a series of packets (each of up to 1024 bytes in size) may be transmitted in a single burst, with a specific packet-limiter separating the individual packets. In this case, the time-slot specified for packet transmission shall be considered as the slot in which the burst transmission is to commence.

  - RMIM shall ensure that the transfer of the packet into the modem’s buffer is completed at least in 20 milliseconds in advance of the time-slot commencement. For this purpose, the time duration for the Tx to the modem buffer shall be computed as follows:

Tx2Modem duration (in milliseconds) = Packet Size (in bits) /                              q				   SerialLine_Baudrate (in kbps) * 1000,

where SerialLine_Baudrate is 57.600 kbps

  - RMIM shall ensure that the packet transmission to the modem is commenced As Late As Possible, while ensuring compliance with the above clause. This requirement is particularly critical on the STCAS side, as the outbound bursts may occupy several time-slots for transmission to the modem and consequent under-utilization of transmission capacity (as the transmission to the modem and OTA transmission cannot be simultaneous).

  - RMIM shall raise the Request To Send line by default in 3 milliseconds in advance of the time-slot commencement.

  - The Request To Send line shall be lowered at the estimated time of completion of the OTA transmission of the packet per-sec, and before the modem starts to transmit the postamble - for this purpose, the time duration for completion of the OTA transmission shall be computed as follows:

Tx duration (in milliseconds) = Packet Size (in bits) /

OTA_Baudrate (in kbps)  * 1000,

where OTA_Baudrate is 19.200 kbps

  - If a previously queued packet is yet to be transferred to the modem and a new packet is received from the SC for the same time-slot, RMIM shall override the packet in its Tx queue with the new one.

  - Once the transfer of a packet to the modem is completed, the corresponding buffer shall be treated as free to ensure that the packet is not re-transmitted.

  - If an outbound packet is received by RMIM later than the commencement time of the specified time-slot, the packet shall be queued for transmission in the next frame-cycle:

    - At the time of receipt of a packet for Tx, a flag associated with the packet shall be marked as QUEUED.

    - Upon initiating the transfer of this packet to the modem, this flag shall be set to TRANSMITTED.

    - At the commencement of each new frame-cycle, all packets marked TRANSMITTED shall be flushed out and those marked QUEUED (in the previous cycle) shall remain available for transmission (in the current cycle).

  - RMIM shall alternate between the two modems for transmission purposes, at the end of each sequence of RadioModemTxSwitchCycles frame-cycles, where RadioModemTxSwitchCycles is a configurable parameter set to 3 by default.

    - RMIM shall provide for prior specification of time-slots that shall be used for    transmission of packets / bursts. It shall be noted that:

    - On the LocoTCAS, a single pre-specified Tx time-slot shall be applicable for Tx of regular packets only, while any random slot may be selected (by Comm Task) for Tx of broadcast packets; The specified slot is however be subject to change upon registration with an approaching station.

    - On the STCAS, one or more pre-specified Tx time-slot(s) shall be applicable for Tx of regular packets only, while any of the STS or SE slots may be selected (by Comm Task) for Tx of broadcast packets. While the pre-selected slots for Tx of regular packets shall be configurable, RMIM shall also be capable of handling dynamic time-slot selection (if bursts are not configurable, Comm Task shall select the optimal slot for each packet).

- Data Reception

  - RMIM shall separate the incoming data stream into packets based on the 4-byte packet delimiter.

  - RMIM shall receive incoming packets on each of the modems that are not faulty and shall hand over the same to the SC for further processing. RMIM shall tag each incoming packet with the following meta-data:

    - The ID of the modem (A or B) that the packet has been received on.

    - A combination of frame-number and a sequence ID.(Notes 1 & 2)

  - Each packet received from the modem shall be passed on to the SC without any processing at the RMIM level.

Notes:

- This frame-number shall be derived independently (without consideration of the frame-number encoded in the packet) by RMIM from the RTC:

- At 0:0:0 hrs, frame-number = 1

- Every 2 seconds, it is incremented by 2

- The sequence number shall be reset at the commencement of each frame-cycle.

- Diagnostics

- Event Generation

Radio communication protocol between RCC and CPU

Packet structure from VC (Simulator) to Radio Interface (vice versa)

| SOF | SID | DID | FID | F.Num | DL | Data | Data CRC | Frame CRC | EOF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

SOF ---------> Start of Frame --------->1 Byte

SID ---------> Source ID ---------------> 1 Byte

DID ---------> Destination ID ---------> 1 Bytes

FID ---------> Function ID ---------> 1 Byte

F.Num --------->Frame Number---------> 1 Byte

DL ---------> Data Length ---------> 2 Bytes

Data ---------> Data ---------> Max 1024 Bytes

Data CRC ---------> Calculate CRC for Data ---------> 4Bytes

Frame CRC ---------> Calculate CRC for total packet without SOF & EOF ---------> 4 Bytes

EOF---------> End of Frame --------->1 Byte

RCC to CPU Packets

List of packets		FID

- Periodic Data Pkt		0 x 01

- Frame No  			0 x 02

- Data Req Pkt  			0 x 03

- Radio In Data  		0 x 06

- SID (RCC1/RCC2)		0 X 11 / 0 X 12

- DID (CPU1/CPU2)		0 X 21 / 0 X 22

Periodic Pkt: Transmitting at every 200ms

Data Length=3

Data [0] = Radio1/Radio2

Data [1] = Radio-1/0 - Status ok / Status Not ok

Data [2] = FDMA Configuration checksum (sum of all Byte[1] to Byte[29] in FDMA pkt)

Frame No: Transmitting at every 2000ms even PPS

Data Length=6

Data [0] = GPS1/GPS2

Data [1] = GPS - 1/0 - Status ok / Status Not ok

Data [2 - 5] = Frame Number

Data Req Pkt:

Data Length=2

Data [0] = Radio1/Radio2

Data [1] = Req Type - 1: Auth Req Pkt, 2: Emerg Pkt, 3: Reg Pkt

Radio In Data:

Data Length=1+Rx Data Length

Data [0] = Radio1/Radio2

Data [1- 1023] = Rx Data

CPU to RCC Packets

List of packets		FID

- FDMA Data  			0 x 91

- Slot Data  			0 x 92

- Auth Req Data		0 x 93

- Emer Pkt Data			0 x 94

- Reg Pkt Data 			0 x 95

- SID (CPU1/CPU2)		0 X 21 / 0 X 22

- DID (RCC1/RCC2)		0 X 11 / 0 X 12

FDMA Data: If FDMA checksum not match, then this pkt will trigger

Data Length=30

Data [0] = Radio1/Radio2

Data [1- 2] = Sec1 Fo freq start	                Numerical Time Slot Value in msec

Data [3- 4] = Sec1 Fo freq stop

Data [5- 6] = Sec2 Fo freq start

Data [7- 8] = Sec2 Fo freq stop

Data [9- 15] = Fo freq

Data [16- 22] = Reg Tx Freq	     ASCII Value

Data [23- 29] = Reg Rx Freq

Time Slots:

Data Length=7

Data [0] = Radio1/Radio2

Data [1- 2] = Reg Pkt Time in msec

Data [3- 4] = Auth Pkt Time in msec

Data [5- 6] = Emerg Pkt Time in msec

Auth Req Pkt:

Data Length=1 + Auth Req Pkt Length

Data [0] = Radio1/Radio2

Data [1-1023] = Auth Req Pkt

Emerg Pkt:

Data Length=1 + Emerg Pkt Length

Data [0] = Radio1/Radio2

Data [1-1023] = Emerg Pkt

Reg Pkt:

Data Length=1 + Reg Pkt Length

Data [0] = Radio1/Radio2

Data [1-1023] = Reg Pkt

# 3. Comm Task - Common Features

## Interaction with KMS

- Obtaining the Authentication Keys from the KMS task running in the CPU-C; the keys shall be stored in a KeyStore and used for generation of the CBC-MAC field for outgoing packets and for re-generating the CBC-MAC field from the contents of the incoming packets. Other parameters required for the generation of CBC-MAC values shall be made available by SIL4.

## Management of Sessions

In the following, the term peer entity referred to shall be interpreted as follows:

- In STCAS, each LocoTCAS entity that is successfully admitted / registered

- In LocoTCAS, this shall be the STCAS entity with which registration has been completed successfully.

Notes:

- STCAS shall maintain one session for each Loco in its territory, while LocoTCAS shall, at any point in time, maintain a single session - with the specific STCAS it is (imminently going to be) registered with.

### Session Descriptors

This session specific data structures shall contain:

- The random number generated at the commencement of the session (by SIL4) for possible re-use in

  - transmitting Access Request packets until the Access Authority packet is received

  - transmitting Access Authority packets until a Loco2STCAS Regular packet is received by STCAS

- In STCAS, Comm Task will store the Loco Random Number and the Station Random Number from the BB message related to Access_Auth received from SIL4.

- In LocoTCAS, Comm Task will store the Loco Random Number from the BB parameters related to the outgoing Access_Request packet; and the Station Random Number from the Access_Auth packet received from STCAS.

- Session Key used for generation of CBC-MAC during the life-time of the session

- A timer for determining radio communications failure

- A timer for determining session timeout (also referred to as “random number timeout”)

- Statistics pertaining to the session; these statistics shall be persistent across all sessions

### Communications Failure Timer

- The Comm Task shall determine the integrity of radio communications with the peer entity, while in the Communication Mandatory Zone (based on Session Key being valid). A Communications Failure Timer (for the configured duration) shall be restarted upon processing a (valid) regular packet from the peer; Upon this timer expiring, SIL4 shall be notified of Radio Communications Failure for appropriate actions.

- All packets received after communications or session timeouts shall be passed on SIL4

### Session Timer

- The session timeout shall be based on the configuration parameter “Random Number timeout” (Ref: Annexure A7/7 for STCAS and Annexure A5/11.8 for LocoTCAS)

- The Comm Task shall restart a Session Timer upon determination of Radio Communications failure with the peer entity; Upon expiry of this timer, the Session Key applicable for this peer session shall be discarded; Since any further exchange of Regular packets with the peer is permissible (feasible) only after exchanging Random Numbers, Comm Task in STCAS shall notify SIL4 (of Session Timeout) which shall in turn deregister the corresponding LocoTCAS entity.

- Upon session timeout, LocoTCAS shall generate a new random number and (continue to) transmit Block-Section / Access Request packets.

## Packet Transmission

- TDMA_Slot on which the packet is to be transmitted shall be specified by SIL4 for all packet types (Note 1)

- In case of burst transmission from STCAS however, the TDMA_Slot indicated by SIL4 is to be considered as the slot in which the burst transmission is to commence.

- Frame-number shall be maintained by Comm Task;

Notes:

- Random time-slot selection logic for outgoing block-section packets shall be implemented in SIL4;

Implementation Notes:

  - In each scheduling cycle, the (entire set of) BB message(s) from the SIL4 task shall be processed and the relevant outgoing radio packets generated.

  - The Comm Task shall generate the packets in conformance with the BB messages without consideration of the commencement time for OTA transmission (note 1).

  - The BB messages are structured so as to provide all the information required to be encoded into the radio packets, including, where necessary, “control flags” indicating the applicability or otherwise of certain fields.

  - CRC Insertion. For each outgoing packet, the CRC shall be generated based on the payload and appended to the same

  - CBC-MAC Insertion. For each outgoing regular and Access_Authority packet, the CBC_MAC shall be generated based on the payload and inserted in the appropriate field.

  - Packet / Byte-count Statistics shall be maintained:

    - Total number of Packets / Bytes Transmitted / Received

    - Total number of erroneous Packets / Bytes Received

Notes:

- The RMIM component (in CPU-C) shall implement the necessary logic to:

    - Buffer the packets until the appropriate time for transferring the packet to the modem, if necessary in the next frame-cycle.

    - Overwrite a packet that might already be queued up for the same time-slot

## Packet Reception

  - Each incoming packet (received from RMIM) shall be accompanied by the following meta-data:

    - The ID of the modem (A or B) that the packet has been received on;

    - A RxPktSeqNumber (combination of frame-number and sequence ID).

  - An internal array of variables “FrameSeqProcessed” shall be maintained to track whether a sequence ID is processed or not (within the frame-cycle).

  - For each incoming packet, the CRC value shall be calculated from the payload (all the bytes of the packet except the last 4 bytes that constitute the CRC field); if the calculated CRC value matches with the CRC field encoded in the packet, the packet is taken up for further processing; in this case, the corresponding FrameSeqProcessed variable shall be set to avoid processing the same packet received from the other modem (notes 1,2  & 3)

  - If the above CRC check fails, the current packet is discarded;

  - Once the CRC validation is completed for a packet, the frame-number shall be verified:

    - A packet with the frame-number field set to the current or the previous frame-number shall be taken up for further processing

    - A packet with any other frame-number shall be discarded with appropriate updates to statistics

  - All the information decoded from the packet shall be populated into the appropriate data structures that are then passed on as BB messages to SIL4. Where applicable, control flags shall be set to the appropriate values to indicate the availability or otherwise of certain fields.

Notes:

- In STCAS, only one Regular packet is expected from each Loco within a frame-cycle; similarly, only one Block-Section / Access-Request packet is expected from each Loco; Comm Task shall discard a duplicate packet if one is encountered within a frame-cycle;

- In LocoTCAS, only one Block-Section packet is expected from another Loco; any duplicate packet, if encountered within a frame-cycle, shall be discarded.

- In LocoTCAS, a series of Regular packets may be received (in one or more bursts); as each packet is decoded, all packets except those addressed to self Loco shall be discarded;

## Modem Management

- Initialization / configuration of the modems

- Monitoring / reporting the health of modems

- Monitoring / reporting RSSI

- Power Settings - based on configuration [potentially different levels for the block-section and the station area]

# 4. Comm Task (STCAS) Requirements

## Initialization / Configuration

The following parameters shall be initialized at system initialization time:

  - the FDMA frequencies allocated to the station - fsx for transmission and fmx for reception); these parameters are valid for the lifetime of the system;

  - the broadcast frequency (f0);

  - The configuration of bursts: one (or more) set(s) of contiguous TDMA time-slots that STCAS can transmit Station2Loco Regular packets in;

  - The pool of TDMA slots that this STCAS can assign to LocoTCAS entities in its territory.

  - De-registration timeout period configured separately for absolute and auto block sections.

## Packet Transmission

### Access_Authority packets

- For Access_Authority packets, the CBC-MAC field shall be generated based on the payload and other session-specific parameters specified in the BB message from SIL4 - this CBC-MAC field shall be appended to the payload.

- The (1632-bit) CRC value shall be computed on the extended payload (subsequent to appending CBC-MAC) and appended to the same.

- A configuration parameter NumAccAuthPktsInTS determines if two Access_Authority packets can be transmitted in succession as a burst within a single time-slot (one of STS-1 through STS-4). This parameter shall be set to 1 by default.

### Station2Loco Regular Packets

The following shall be provided for:

1.    A Regular packet may contain information for only one Loco.

a.    The 32-bit CBC-MAC shall be generated for the entire payload up to but excluding the CBC-MAC field itself

b.     The 32-bit CRC field shall be generated for each packet based on the payload up to and including the CBC-MAC field and appended to the same.

2.     All profiles corresponding to a single routing table (ToC) entry shall be included in a single packet:

a.     Whenever MA is being extended, all applicable track profiles shall be transmitted (this shall be initiated by SIL4 task)

b.     Otherwise, only the MA profile shall be transmitted

c.     A packet may extend up to MaxRadioPacketSize (configurable parameter, default 732 bytes).

3.     If multiple sets of profiles (each corresponding to a different route) are to be transmitted at the same time to the Loco, each of the profiles shall be transmitted as separate packets in one or more burst(s) (Note 1 below)

- each profile in a single burst may refer to the same LAST_REF_RFID

- each profile in a single burst may refer to different DIST_PKT_START values;

- packets in different bursts may refer to the same or different LAST_REF_RFID, to provide for the possibility of the Loco having reported a new RFID tag in the time interval between bursts;

- The REF_PROF_ID field shall be maintained on a per-Loco basis, incremented by one for each packet containing any sub-packet other MA

4.      Consecutive packets in a burst shall be separated by the packet delimiter (the 4-byte pattern 0xA5-0xC9-0xA5-0xC9).

5.      The entire transmission capacity in one half of a frame-cycle of MaxRadioBurstSize (configurable, default of 1288 bytes) may be taken up by one or more bursts. (Note 2 below)

Notes:

1)     While STCAS shall transmit multiple profiles, the merger of the same shall be handled at the  LTCAS end.

2)     Out of the actual transmission capacity of 1308 bytes, 20 bytes are provisioned for pre- and post- ambles and bit-stuffing

## Packet Reception

Each incoming packet with a valid CRC and frame-number shall be taken up for further processing as described below.

### Block-section packets

A block-section packet shall be subjected to the following checks, treated as Access_Request (note 1), fully decoded and the extracted parameters are all passed on to SIL4:

- Packet Length (should be BLK_SEC_PKT_SIZE = 2729 bytes)

- Source Loco Version (should be 2, “Kavach Specification 4.0”)

- Train Length shall not be zero

- Train Speed shall be between 0 and 400 kmph

- MOVEMENT_DIR shall be 0 (Undefined), 1 (Nominal) or 2 (Reverse)

- EMERGENCY_STATUS shall not be any of the spare values

- Aproaching_Station_ID shall be self-Station ID

- TIN shall be a valid value - between [TIN_MIN = 1, TIN_MAX = 126251] (TIN_MIN and TIN_MAX shall be configurable parameters)

### Loco2Station Regular Packets

A regular packet shall be subjected to validation of the following fields - packets that pass these checks are passed on to SIL4:

- Packet Length (should be LOCO2STN_REG_PKT_SIZE = 2829 bytes)

- CBC-MAC

- Source Loco Version (should be 2, “Kavach Specification 4.0”)

- Train Length shall not be zero

- Train Speed shall be between 0 and 400 kmph

- MOVEMENT_DIR shall be 0 (Undefined), 1 (Nominal) or 2 (Reverse)

- TIN shall be a valid value - between [TIN_MIN = 1, TIN_MAX = 126251] (TIN_MIN and TIN_MAX shall be configurable parameters)

- Brake_Applied shall be a valid value (i.e., shall not be any of the spare values)

Notes:

- Block-section packets from an already registered LocoTCAS entity shall be silently discarded (Comm Task shall determine the registration status of each Loco by the session key: if a valid key is active, the Loco shall be considered as registered)

# 5. Comm Task (LocoTCAS) Requirements

## Initialization / Configuration

- The following parameters shall be initialized at system initialization time:

  - the broadcast frequency (f0);

- The following parameters shall be initialized based on the parameters received from STCAS at the time of registration - these parameters shall be re-initialized at the time of registering with each STCAS encountered during the mission:

  - the FDMA frequencies - fmx for Transmission and fsx for reception;

  - the TDMA time-slot for transmission of Loco2 Station Regular packets

- The switching to the new frequencies indicated by the (newly) registered STCAS shall be effective from the time the next frequency-switch is due (in the current frame-cycle) (Note 1 below).

Notes:

- While the Loco is NOT registered with any station, the frequency-switching may be suspended (i.e., the modems may remain tuned to f0)

## Packet Transmission

### Block Section Packets

- SIL4/MS initiates the Tx of these packets every 2 seconds, only while the Loco is in the block section or under upon restart (or similar conditions) in the station area (note 4).

- While in the station area (as signaled by SIL4), Comm Task shall shall discard incoming block-section packets

- SIL4 shall implement logic to pick a random slot on which a block-section packet is to be transmitted:

MBSx = Random Number in the range [MBS-1 to MBS-6] for Nominal DoM

MBSx = Random Number in the range [MBS-7 to MBS-12] for Reverse  DoM

### Loco2Station Regular Packets

Once a session with an (approaching) STCAS is established, LocoTCAS SIL4 initiates the Tx of Regular packets (note 5 & 6).

- Comm Task shall encode the outgoing Regular packet with the parameters provided by SIL4 task

- Comm Task shall calculate CBC-MAC on the payload, based on the session key generated as part of the registration process this CBC-MAC shall be appended to the payload to form the extended payload.

- Comm Task shall also calculate the CRC on the extended payload and append the same to it.

- Comm Task shall Tx the Regular packet through the RMIM. The TDMA time-slot assigned by STCAS as part of the session establishment process shall be used for this purpose.

## Packet Reception

### Block Section Packets

- Comm Task shall implement logic to discard all block section packets received with the TIN field set to any value other than (note 1):

  - The current (block-section) TIN that self-Loco is in

  - The TINs of any of the Adjacent Lines (as signaled by SIL4 whenever such info is received from STCAS)

- All parameters decoded successfully from valid incoming block section packets shall be passed onto SIL4 for further processing (Block-Section Collision Avoidance logic)

Notes:

- This is to avoid processing packets received from Locos in block-section lines that are distant (non-adjacent) physically but within radio range.

- The interface between SIL4 and Comm Task shall provide for SIL4 to signal the presence of the Loco in the Station Area or the Block Section;

- The transmission of regular packets may be initiated multiple times in a frame-cycle; in all such instances, Comm Task shall generate the packets and hand the same over to RMIM (which will transmit the latest version received prior to the commencement of the TDMA time-slot).

- LocoTCAS shall cease to transmit Loco2Station Regular packets subsequent to:

  - Entering an absolute block section from a station area

### Access_Authority packets

An Access_ Authority packet shall be subjected to the following checks, fully decoded and the extracted parameters are all passed on to SIL4:

- Packet Length (should be ACCESS_AUTH_PKT_SIZE = 22 bytes)

- SOURCE_STN_ILC_IBS_ID shall match with the Approaching StationID

- SOURCE_STN_ILC_IBS_ID_VERSION (should be 2, “Kavach Specification 4.0”)

- DEST_LOCO_ID shall match with self-LocoID

- Allotted_FDMA_Freq shall match one of the pairs configured

- Allotted_TDMA_Timeslot shall be in the valid range [M-1 through M-50]

Notes:

- The Allotted_FDMA_Freq and Allotted_TDMA_Timeslot values shall be stored in internal variables to be used for subsequent transmission of Loco2Station Regular packets.

### Station2Loco Regular Packets

- Comm Task shall decode and process the Station2Loco Regular packets as per packet structures detailed in reference (note 1)

- The CRC shall be validated and the packet discarded if this check fails.

- Comm Task shall decode and validate the “Pkt Header” as follows:

  - PKT_TYPE field shall be 1001

  - PKT_LENGTH shall not exceed MaxRadioPacketSize (configurable parameter, by default 128)

  - FRAME_NUM shall corresponding to either the current frame-cycle or the previous one

  - SOURCE_STN_ILC_IBS_ID shall match with the ID of the STCAS registered with

  - SOURCE_STN_ILC_IBS_VERSION shall be 2 “Kavach Specification 4.0”

  - Header_FILL_ZEROs (2) shall be skipped

  - DEST_LOCO_ID shall be extracted - if this does not match self LocoID, the packet shall be discarded (processing shall continue with the next packet)

  - For a packet with matching DEST_LOCO_ID,  the following parameters are then decoded / extracted:

    - The CBC-MAC field is verified and the packet is discarded if this check fails

    - REF_PROF_ID

    - LAST_REF_RFID

    - DIST_PKT_START

    - PKT_DIR

- Comm Task shall disaggregate the packet - each of the sub-packets following the packet header shall then be processed (notes 3 & 4)

  - Movement Authority (MA) sub-packet. It is to be noted that two types of MA are provided for: FS-MA applicable to authorize a Loco to proceed in the FS (Full Supervision) mode (wherein LocoTCAS is responsible for movement supervision) and OS-MA applicable to authorize a Loco to proceed in the OS (On-sight) mode (wherein both LocoTCAS and the LP are jointly responsible for movement supervision) subject to required conditions being met. It is also to be noted that the MA is with reference to the location of the LAST_REF_RFID tag.

  - Static Speed Profile containing a (series of) of LMs specifying the Speed Limits and related details corresponding to different sections of the track.

  - Gradient Profile containing a (series of) of LMs specifying the Gradient related details corresponding to different sections of the track.

  - LC Gate Profile containing a (series of) of LMs specifying the LC Gate related details for the upcoming sections of the track.

  - Turnout Profile containing details pertaining to one or more Turnouts (Commencement Point, Distance and Speed Restriction)

  - Tag Linking Profile containing an array of RFID Tag-IDs and their locations that the Loco should expect to read on the route.

  - Track Condition Profile containing details such as the presence of a Radio Hole in the route.

  - TSR Profile containing details pertaining to Temporary Speed Restrictions.

Notes:

- Reference: Radio Pkts v2.0, Tab: RadioPktsv2.0

- All sub-packets are optional elements in a particular packet and so the packet may contain any combination of the sub-packets, in any order.

- All sub-packet are variable in size. However, each sub-packet shall contain an integral number of bytes - if so warranted, the “payload” of the sub-packet will be padded with between [1,7] bits such that the sub-packet-size in bits is an integral multiple of 8.

- Each sub-packet is restricted in size (MAX_SUB_PKT_SIZE = 128 bytes)

- It shall be noted that each packet is self-contained by itself and hence shall be processed independently.

- Based on signaling by SIL4, Comm Task in LocoTCAS shall cease to process Station2Loco Regular packets subsequent to:

  - Encountering an RFID tag set to “Communication NOT Required” in an outbound absolute block section area

  - Encountering an RFID tag (border tag) in an outbound auto section with the “Approaching” Station ID set to a different one than the dispatching station.

# KMS

- The KMS component (common to both LocoTCAS and STCAS) shall run on the CPU-C

- KMS shall communicate with the central KMS server via GPS/GPRS interfaces

- Once a set of Authentication Keys are obtained from the central server, the same are passed on to the Comm Task of the SC via the RBB.

# Tx / Rx of Packets by LocoTCAS

| Loco Location | Tag Indications | Tx | Rx (Processing) |
| --- | --- | --- | --- |
| Abs Block Section (Inside CMZ of Dispatching Station) | SectionType (in DoM) = Absolute Block; CM (in DoM) = Required | Block-Section Pkts AND Regular Pkts (to dispatching station) | Block-Section Pkts AND Regular Pkts (from dispatching station) |
| Abs Block Section (Outside CMZ) | SectionType (in DoM) = Absolute Block; CM (in DoM) = Not Required | Only Block-Section Pkts | Only Block-Section Pkts; Discard any Regular Pkt; |
| Abs Block Section (Inside CMZ of receiving station) | SectionType (in DoM) = Abs Block; Station-ID (in DoM) = XXX; CM (in DoM) = Required | Block-Section Pkts; AND (after Registration) Regular Pkts | Block-Section Pkts AND Regular Pkts (from receiving station) |
| Auto Section (Always in CMZ - with either dispatching or receiving station) | SectionType (in DoM)= Auto Block (Note 3) | Block-Section Pkts AND Regular Pkts | Block-Section Pkts AND Regular Pkts |
| Station Area | SectionType (in DoM) = Station Section | Regular Pkts; Stop Tx of Block-section Pkts; | Discard Block-Section Pkts; Process Regular Pkts; |

Notes:

- In an outbound block section, if Communication Required (in DoM) is set to “Not Required”, LocoTCAS shall stop Tx’ing Regular Packets (to the dispatching station) - eventually, STCAS shall timeout and de-register the Loco; Loco shall continue to Tx block-section packets with Approaching Station ID set to Station-ID (in DoM);

- Loco shall always set the Approaching Station ID in outgoing block-section packets to the Station-ID (in DoM) from the most recently read RFID tag;

- LocoTCAS shall process an incoming block-section packet only if the TIN in such a packet is found in “Adjacent Line TINs” as received from STCAS or as read from an RFID tag.

- When STCAS determines that a registered LocoTCAS is dispatched into an absolute block section (i.e., it has crossed the LSS with OS- or FS- MA), it shall cease to respond to any Access Request packets received from it.

- In an Auto Section, Communications shall always be Mandatory with either the dispatching or approaching station - Loco can be in communication with only one station at a time, depending on the Station-ID (in DoM) field of the RFID Tag;

- CMZ: Communications Mandatory Zone

# 7. protEm

- The protEm tool shall be updated with the TCAS 4.0 radio packet decoding routines, while retaining the routines for decoding of TCAS 3.2 packets; the different versions of packets shall be distinguished on the basis of the PKT_TYPE field.

- DB shall be enhanced as follows:

  - Each of the following packet types shall be represented by a corresponding table:

    - Block-Section / Access_Request

    - Access_Authority

    - Loco2Station Regular

    - Station Emergency

  - The Station2Loco Regular packet shall be represented by a set of tables corresponding to each sub-packet:

    - Movement Authority

    - Static Speed Profile

    - Gradient Profile

    - LC Gate Profile

    - Turnout Profile

    - Tag Linking Info

    - Track Condition Data

    - TSR Profile

- Multiple Stations

  - Provision shall be made for multiple stations transmitting to or receiving from the same or different Locos in a single frame-cycle (frame-cycles being common to all entities in the network)

- Station2Loco Regular packets

  - Each packet shall be destined to only one Loco (unlike in 3.2 or earlier versions of 4.0)

  - Provision shall be made for each packet to contain one or more sub-packets

  - Provision shall be made for a Station transmitting multiple packets to the same or different Locos within a single frame-cycle

  - Provision shall be made for a Station transmitting multiple sub-packets of the same type to the same Loco (in the same frame-cycle); for example, a packet may contain MA, while 2 more packets may contain Tag Linking Info - all meant for a single Loco